# Capacity Expansion Test 2 Summary

Generated: 2026-05-21 16:15:35

# Summary

This document summarizes the thermal-only greenfield capacity expansion test in test/capacity_expansion_test_2. The run uses Geothermal, Coal, CCGT, and CT candidates, hourly demand from doc/expansion_data, non-served energy at 9000 USD/MWh, and a Julia/JuMP model solved with HiGHS. The generated result reproduces the thermal case from 03-Basic-Capacity-Expansion.ipynb.

# Prompts Used

## Scenario prompt

Create a solver-ready scenario config for the basic capacity expansion.

Start with the thermal-only greenfield case. Use Geothermal, Coal, CCGT, and CT to meet hourly demand. Non-served energy is allowed with a high penalty. Keep the model in Julia/JuMP with HiGHS.

Return the scenario as a structured YAML config.

## Code generation prompt

Using the scenario config above, generate a Julia/JuMP script for the thermal-only capacity expansion model.

The script should load the generator and demand data, define capacity, generation, and non-served energy variables, add demand balance and capacity constraints, minimize total cost, solve with HiGHS, and save key outputs.

# Skills Used

| Skill | Use in this test |
| --- | --- |
| scenario_builder | Generated the structured solver-ready YAML scenario config for the thermal-only greenfield capacity expansion case. |
| code_generator | Generated the Julia/JuMP implementation that reads the configured data, builds the HiGHS model, solves it, and writes structured outputs. |

# Generated Files

- scenario_config.yaml (8,418 bytes)

- run_capacity_expansion.jl (13,299 bytes)

- capacity_build.csv (172 bytes)

- hourly_generation.csv (505,836 bytes)

- non_served_energy.csv (112,843 bytes)

- solve_summary.json (459 bytes)

- highs_thermal_greenfield.log (1,084 bytes)

# Scenario Config

## File: scenario_config.yaml

scenario_id: "capacity_expansion_test_2_thermal_greenfield"

created_at: "2026-05-21T00:00:00-07:00"

scenario_name: "basic_capacity_expansion_thermal_only"

source_request: "Create a solver-ready scenario config for the basic thermal-only greenfield capacity expansion case using Geothermal, Coal, CCGT, and CT with non-served energy allowed at high penalty."

model:

model_type: "greenfield_generation_capacity_expansion"

formulation_class: "linear_program"

modeling_stack:

language: "Julia"

package: "JuMP"

solver: "HiGHS"

optimizer_constructor: "HiGHS.Optimizer"

network_representation: "single_bus"

existing_capacity_treatment: "greenfield_no_existing_capacity"

unit_commitment: false

storage: false

transmission: false

objective:

sense: "minimize"

description: "Minimize annualized fixed capacity cost, variable generation cost, and non-served energy penalty over all modeled hours."

expression: "sum(g in G, FixedCost[g] * CAP[g]) + sum(g in G, h in H, VarCost[g] * GEN[g,h]) + sum(h in H, NSECost * NSE[h])"

terms:

fixed_capacity_cost:

variable: "CAP[g]"

parameter: "FixedCost[g]"

units: "USD/year"

variable_generation_cost:

variable: "GEN[g,h]"

parameter: "VarCost[g]"

units: "USD"

non_served_energy_penalty:

variable: "NSE[h]"

parameter: "NSECost"

units: "USD"

sets:

G:

description: "Thermal candidate generation technologies."

source_file: "doc/expansion_data/generators_for_expansion.csv"

source_column: "G"

include_values:

- "Geo"

- "Coal"

- "CCGT"

- "CT"

expected_descriptions:

Geo: "Geothermal"

Coal: "Supercritical Coal"

CCGT: "Natural gas CCGT"

CT: "Natural gas CT"

excluded_values:

- "Wind"

- "Solar"

H:

description: "Hourly demand periods for the planning year."

source_file: "doc/expansion_data/demand_for_expansion.csv"

source_column: "Hour"

expected_count: 8760

resolution_hours: 1.0

input_data_files:

generators:

path: "doc/expansion_data/generators_for_expansion.csv"

format: "csv"

required: true

required_columns:

- "G"

- "Description"

- "Capex"

- "FixedOM"

- "VarOM"

- "HeatRate"

- "FuelCost"

- "WACC"

- "AssetLife"

- "CRF"

- "Annuity"

- "FixedCost"

- "VarCost"

demand:

path: "doc/expansion_data/demand_for_expansion.csv"

format: "csv"

required: true

required_columns:

- "Hour"

- "Demand"

candidate_generator_filter:

mode: "include_only"

technology_id_column: "G"

include_values:

- "Geo"

- "Coal"

- "CCGT"

- "CT"

exclude_values:

- "Wind"

- "Solar"

fail_if_excluded_resource_selected: true

parameters:

demand:

symbol: "Demand[h]"

source_file: "doc/expansion_data/demand_for_expansion.csv"

source_column: "Demand"

units: "MW"

interpretation: "Hourly demand; with one-hour timesteps this is equivalent to MWh required in each period."

fixed_cost:

symbol: "FixedCost[g]"

source_file: "doc/expansion_data/generators_for_expansion.csv"

source_column: "FixedCost"

units: "USD/MW-year"

variable_cost:

symbol: "VarCost[g]"

source_file: "doc/expansion_data/generators_for_expansion.csv"

source_column: "VarCost"

units: "USD/MWh"

non_served_energy_cost:

symbol: "NSECost"

value: 9000.0

units: "USD/MWh"

description: "High penalty for involuntary non-served energy."

timestep_hours:

symbol: "dt"

value: 1.0

units: "hours"

decision_variables:

CAP:

index: ["G"]

domain: "NonNegativeReals"

units: "MW"

description: "New generating capacity built for each thermal candidate."

GEN:

index: ["G", "H"]

domain: "NonNegativeReals"

units: "MWh"

description: "Generation from each thermal resource in each hour."

NSE:

index: ["H"]

domain: "NonNegativeReals"

units: "MWh"

description: "Non-served energy in each hour."

constraints:

demand_balance:

index: ["H"]

expression: "sum(g in G, GEN[g,h]) + NSE[h] == Demand[h]"

description: "Each hour's demand must be met by generation or non-served energy."

generation_capacity:

index: ["G", "H"]

expression: "GEN[g,h] <= CAP[g] * dt"

description: "Hourly generation from each thermal resource cannot exceed installed capacity for the one-hour interval."

nonnegative_capacity:

index: ["G"]

expression: "CAP[g] >= 0"

description: "Capacity build decisions are continuous and nonnegative."

nonnegative_generation:

index: ["G", "H"]

expression: "GEN[g,h] >= 0"

description: "Generation decisions are continuous and nonnegative."

nonnegative_nse:

index: ["H"]

expression: "NSE[h] >= 0"

description: "Non-served energy is continuous and nonnegative."

solver_settings:

solver_name: "HiGHS"

optimizer_constructor: "HiGHS.Optimizer"

model_sense: "Min"

time_limit

... [truncated in document]

# Solve Results

| Metric | Value |
| --- | --- |
| Scenario ID | capacity_expansion_test_2_thermal_greenfield |
| Termination status | OPTIMAL |
| Primal status | FEASIBLE_POINT |
| Objective value | 990,873,978.80 USD |
| Total fixed cost | 355,352,720.00 USD |
| Total variable cost | 629,788,258.80 USD |
| Total NSE | 637.00 MWh |
| Total NSE penalty | 5,733,000.00 USD |
| Peak demand | 4,813.0 MW |
| Modeled hours | 8760 |

## Capacity Build

| Resource | Description | Capacity MW | Fixed Cost USD |
| --- | --- | --- | --- |
| Geo | Geothermal | 0.0 | 0.00 |
| Coal | Supercritical Coal | 0.0 | 0.00 |
| CCGT | Natural gas CCGT | 3,328.0 | 274,227,200.00 |
| CT | Natural gas CT | 1,290.0 | 81,125,520.00 |

# Notebook Comparison

| Metric | Notebook | capacity_expansion_test_2 | Result |
| --- | --- | --- | --- |
| Objective | 990,873,978.80 USD | 990,873,978.80 USD | Match |
| Geo capacity | 0 MW | 0 MW | Match |
| Coal capacity | 0 MW | 0 MW | Match |
| CCGT capacity | 3328 MW | 3328 MW | Match |
| CT capacity | 1290 MW | 1290 MW | Match |
| NSE max | 195 MW | 195 MW | Match |
| NSE total | 0.637 GWh | 0.637 GWh | Match |
| CCGT generation | 22139.4 GWh | 22139.366 GWh | Rounding only |

The notebook and generated script solve the same thermal greenfield LP: minimize fixed capacity cost, variable generation cost, and NSE penalty subject to hourly demand balance and generation capacity limits. The objective and capacity mix match; the small CCGT generation difference is display rounding.

# Conclusion

The scenario config and Julia/JuMP script are consistent with the notebook formulation. The HiGHS solve returned OPTIMAL and produced the expected buildout: 0 MW Geo, 0 MW Coal, 3328 MW CCGT, 1290 MW CT, and 637 MWh total non-served energy.